TAC-157X-L

Our SafeTCap® TAC-157 & TAC-083 are designed  to repair the frame section right before the rear tires at the forward rear spring mount. A version of the TAC-157 is made to fit every cab model 1996-2004 Toyota Tacoma. The TAC-083 fits the extended cab models only. These kits are made from 11 gage steel, CNC cut and Mig welded in our USA plant. The TAC-157 kit includes a 7 gage forward rear spring mount. You will need to remove the fuel tank to install these frame repair kits. If you’re installing the TAC-083 you will need to carefully remove the spring mount and reinstall it once the frame repair kit has been installed.

Caution!  If you are not fully confident doing the installation yourself, have a professional welder install these repair kits, or at least have one inspect your repairs before you finish coat your SafeTCap® installation.

TAC-157X-R

V

- Important!:  Read and understand the “General Instructions for installing SafeTCap® frame repair kits” before you begin.

- If your installing frame repair kits on both sides, make sure to only repair one side at a time. This will help to keep the frame stable when removing the rusted portions, and you will also have the opposite side as a reference if needed.

- We’ve designed these frame repair kits to be installed with the bed of the truck left on. This also helps to keep the frame stable and straight during installation. You can continuously check the gap between the cab and bed to make sure the frame hasn’t shifted or weakened during the install.

- Jack up the rear of the vehicle until you have enough room to remove the fuel tank. Support the vehicle on properly rated jack stands. Putting the jack stands under the frame about 7 inches forward of the rear cab mount on each side works well, and also put two more jack stands under the rear ends of the frame for support. The rear axle must hang free to remove the forward rear leaf spring mount.

- Remove the rear tires and the fuel tank from the vehicle. Store the fuel tank in a safe area away from the repair.

- Remove the forward spring bolt, and pry the spring from the mount. This is a good time to replace the spring bushing and bolt.

Rear

Front

Measure the forward spring mount bolt hole center to rear spring shackle mount bolt center. Diagram below.

Ref measurement

TAC-157

Cut the spring mount from the frame along with any rusted lower section. Do not cut more than half of the lower portion of frame. After cutting spring mount off, slide the SafeTCap kit onto the frame and position it using the measurement taken earlier,  mark the entire outline of the frame kit, This will help so you don’t remove too much of the original frame. Note: For a proper install you should allow the SafeTCap®  kit to overlap at least 1 inch of the original frame all the way around.

- If you’re installing the TAC-083, cut the forward rear spring mount from the frame as close to the mount as possible without cutting into it. Note: The original spring mount must be intact and cleaned for re-use. The spring mount will need to be welded to the TAC-083 after its installed.

- Grind your original frame to clean, bare metal all along the marked outline, to allow for a strong solid weld.

Figure B

Slide the SafeTCap® kit onto the frame and position using the measurements taken earlier. Clamp onto the frame and close any gaps. See figure: B

- For TAC-083 kit: place the original spring mount onto the frame kit using measurements taken earlier and tack weld to the frame once satisfied with the position.

- When satisfied with the position of the frame kit, tack weld it to the frame till its securely in place. Re-check measurements of the spring mount, and check the fit of the spring to the new mount if installing the TAC-157.

- Weld the frame repair kit solid to the frame. We suggest welding 1”- 1½” beads every 3” inches making sure not to overheat any one area. For TAC-083 install: weld the original spring mount solid to the frame kit.

- Reinstall the leaf spring the coat the frame with a rust inhibiting primer or undercoat.

- Reinstall fuel tank and rear tires. Lower to the ground and check the gap between cab and bed…

Pic Shows TAC-157X-L   Driver side

We Make the Kits, that Fit!